Firewall Rule Design and Network Segmentation Concept for a Bank Using Site-to-Site VPN

Internship project report

Prepared by: Meleng Jemsimbom Collins, IT Intern

Company: Bridge Technologies Solutions, Douala (Bonamoussadi), Cameroon

Date: 28 September 2026

Tools: GNS3, VMware Workstation Pro, FortiGate (FortiOS 7.0.14), Cisco IOS 12.4

# 1. Introduction

My supervisor asked me to design and build a simulated bank network that shows how firewall rules, VLAN segmentation and a site-to-site IPsec VPN fit together. The lab has a Head Office and a Branch Office, joined by an encrypted tunnel across a simulated Internet. Everything runs in GNS3, with the GNS3 VM installed inside VMware Workstation Pro.

This report covers what I built, why I made each design choice, how I tested it, and what went wrong on the way. I have tried to be direct about the limits of the lab, because a few of them change how far the results can be trusted. Those are collected in section 11.

# 2. Lab environment and device roles

My supervisor gave me four images. Not all of them were needed, and I chose the role of each one instead of forcing every image into the design.

| Image | Used as | Reason |
| --- | --- | --- |
| fortios.qcow2 | FGT-HQ and FGT-BR (two instances) | Firewall, VLAN gateway and VPN endpoint at each site. Using the image twice gave me a real FortiGate to FortiGate tunnel. |
| c2691 (Cisco 2691) | ISP-R1 | Stands in for the Internet. It only needs two addressed interfaces and static routes, so the lighter router was enough. |
| c7200 (Cisco 7200) | Not used | Nothing in the design needed a second router. I left it as a spare. |
| i86bi-linux-l2 (IOU) | Not used | It needs an IOU licence and I did not have one. I used the GNS3 built-in Ethernet switch for HQ-SW1 and BR-SW1 instead. |

The FortiGate is the default gateway for every VLAN. I did not put a router in front of the VLANs, because inter-VLAN routing on a router would let that traffic skip the firewall. With the FortiGate as gateway, every packet that moves from one VLAN to another goes through a policy check.

For end hosts I started with VPCS nodes, which can only send ping. Later I replaced MGMT-PC, APP-SRV, DB-SRV and BR-STAFF-PC1 with Alpine Linux Docker containers so I could send real HTTPS traffic. The other hosts stayed as VPCS.

## Device inventory

| Device | Role | Platform | Version |
| --- | --- | --- | --- |
| FGT-HQ | HQ firewall, VLAN gateway, IPsec endpoint | FortiGate-VM64-KVM | FortiOS 7.0.14, build 0601 |
| FGT-BR | Branch firewall, IPsec endpoint | FortiGate-VM64-KVM | FortiOS 7.0.14, build 0601 |
| ISP-R1 | Simulated Internet router | Cisco 2691 (Dynamips) | IOS 12.4(13b) |
| HQ-SW1, BR-SW1 | Layer 2 switches | GNS3 Ethernet switch | n/a |
| MGMT-PC, APP-SRV, DB-SRV, BR-STAFF-PC1 | Hosts for real traffic | Docker, Alpine Linux | Alpine 3.24 |
| STAFF-PC1, ATM-PC1, DMZ-WEB, GUEST-PC1, BR-ATM-PC1 | ICMP test hosts | VPCS | n/a |

# 3. Topology

Figure 1. Final lab topology, redrawn from the GNS3 project.

The table below lists every link in the lab. The switch trunk ports carry all HQ VLANs tagged (802.1Q) to the firewall, and I set their native VLAN to 999, which is unused, as a small hardening step.

| Device A | Interface | Device B | Interface | Purpose |
| --- | --- | --- | --- | --- |
| FGT-HQ | port1 | ISP-R1 | Fa0/0 | HQ WAN link |
| FGT-BR | port1 | ISP-R1 | Fa0/1 | Branch WAN link |
| FGT-HQ | port2 | HQ-SW1 | Ethernet0 | HQ trunk, VLANs 10 to 70 |
| FGT-BR | port2 | BR-SW1 | Ethernet0 | Branch trunk |
| HQ-SW1 | Ethernet1 to 7 | MGMT-PC, STAFF-PC1, APP-SRV, DB-SRV, ATM-PC1, DMZ-WEB, GUEST-PC1 | eth0 | HQ access ports, one VLAN each |
| BR-SW1 | Ethernet1, 2 | BR-STAFF-PC1, BR-ATM-PC1 | eth0 | Branch access ports |

# 4. IP addressing plan

I used private IPv4 space and made the third octet match the VLAN ID, so an address tells you its VLAN at a glance. HQ is 10.10.0.0/16 and the Branch is 10.20.0.0/16. Keeping each site inside one /16 means the VPN needs only one pair of selectors, while the firewall policies stay strict per subnet.

| VLAN | Name | Network | Gateway | Trust level | Test host |
| --- | --- | --- | --- | --- | --- |
| 10 | HQ-MGMT | 10.10.10.0/24 | 10.10.10.1 | Critical | MGMT-PC .10 |
| 20 | HQ-STAFF | 10.10.20.0/24 | 10.10.20.1 | Medium | STAFF-PC1 .10 |
| 30 | HQ-SERVERS | 10.10.30.0/24 | 10.10.30.1 | High | APP-SRV .10 |
| 40 | HQ-DB | 10.10.40.0/24 | 10.10.40.1 | Highest | DB-SRV .10 |
| 50 | HQ-ATM | 10.10.50.0/24 | 10.10.50.1 | Restricted | ATM-PC1 .10 |
| 60 | HQ-DMZ | 10.10.60.0/24 | 10.10.60.1 | Low | DMZ-WEB .10 |
| 70 | HQ-GUEST | 10.10.70.0/24 | 10.10.70.1 | Untrusted | GUEST-PC1 .10 |
| 100 | BR-MGMT | 10.20.100.0/24 | 10.20.100.1 | Critical | none placed |
| 110 | BR-STAFF | 10.20.110.0/24 | 10.20.110.1 | Medium | BR-STAFF-PC1 .10 |
| 120 | BR-ATM | 10.20.120.0/24 | 10.20.120.1 | Restricted | BR-ATM-PC1 .10 |

| Link | Network | Addresses |
| --- | --- | --- |
| HQ WAN | 203.0.113.0/30 | ISP-R1 Fa0/0 .1, FGT-HQ port1 .2 |
| Branch WAN | 198.51.100.0/30 | ISP-R1 Fa0/1 .1, FGT-BR port1 .2 |
| IPsec tunnel | 172.31.255.0 range | HQ-to-BR 172.31.255.1/32, BR-to-HQ 172.31.255.2/32 |

The two WAN networks come from the RFC 5737 documentation ranges, so they cannot clash with real addresses. The tunnel interfaces use /32 masks because FortiOS refused a /30 on a route-based tunnel interface ("A tunnel IP must have a mask of 255.255.255.255"). Each end is therefore a separate host address instead of two ends of one subnet.

Routing is all static. ISP-R1 has one route to 10.10.0.0/16 via FGT-HQ and one to 10.20.0.0/16 via FGT-BR. Each FortiGate has a default route via ISP-R1, plus a route for the other site's /16 that points at the tunnel interface (see section 7).

# 5. Network segmentation

The idea behind the segmentation is that each group of devices gets its own VLAN, and a rule has to be written before any VLAN can talk to another. Anything without a rule is dropped by the implicit deny at the bottom of the FortiGate policy list.

| VLAN | What it holds | Why it is separate |
| --- | --- | --- |
| Management | Admin workstation | Only place allowed to administer the firewall and infrastructure. |
| Staff | Employee PCs | Users are the most likely source of mistakes and malware, so they should not touch data stores directly. |
| Servers | Application servers | Business logic sits between users and data. |
| Database | Databases | Highest value target. Only the application tier may connect. |
| ATM | ATM and transaction devices | They sit in public places and can be tampered with. |
| DMZ | Internet-facing web server | Exposed to the Internet, so it must not be a way into the inner network. |
| Guest | Guest devices | Untrusted. Internet access only. |
| Branch VLANs | Branch management, staff, ATM | Same reasoning as HQ, reached only through the VPN. |

## Allowed and denied flows

| Source | Destination | Service | Result | Reason |
| --- | --- | --- | --- | --- |
| HQ Staff | HQ Servers | HTTPS | Allow | Staff use the banking application |
| HQ Staff | HQ Database | Any | Deny | No direct database access for users |
| HQ Servers | HQ Database | MySQL | Allow | Only the application tier reads and writes data |
| HQ ATM | HQ Servers | HTTPS | Allow | ATM transactions |
| HQ ATM | HQ Database | Any | Deny | ATMs never touch data stores |
| HQ Management | Servers, DB, ATM, DMZ | SSH, HTTPS | Allow | Administration |
| HQ Staff | Internet | HTTP, HTTPS, DNS | Allow, NAT | General web access |
| HQ Guest | Internet | HTTP, HTTPS, DNS | Allow, NAT | Guest web access |
| HQ Guest | Any internal VLAN | Any | Deny | Guest isolation |
| Internet | HQ DMZ | HTTPS | Allow | Public web portal |
| HQ DMZ | Any internal VLAN | Any | Deny | Stops a hacked web server from spreading inward |
| HQ Staff | Branch Staff | HTTPS | Allow, VPN | Collaboration between sites |
| Branch Staff, ATM | HQ Servers | HTTPS | Allow, VPN | Branch uses the core application at HQ |
| HQ Management | Branch Management | SSH, HTTPS | Allow, VPN | Central administration |
| Branch Staff | HQ Database | Any | Deny | Branch never reaches data directly |
| HQ Guest | Any Branch VLAN | Any | Deny | Isolation holds across the VPN |
| WAN | Firewall's own interfaces | Any | Deny | Default local-in deny |

# 6. Firewall policies

Every policy below has the action accept, logging set to all sessions, and NAT off unless the table says otherwise. NAT is only on for the two policies that send traffic out to the Internet. Internal traffic is not translated, because I want the real source address in the logs.

Each service list also contains ALL_ICMP. This is a lab accommodation, not a design choice. VPCS can only send ping, so without ICMP in the list I could not have tested any of these policies. In a real ruleset I would remove ICMP from every rule except where monitoring needs it.

## HQ policies (FGT-HQ)

| # | Name | Interfaces (in, out) | Source, Destination | Service | NAT |
| --- | --- | --- | --- | --- | --- |
| 1 | Staff-to-AppServer | VLAN20-STAFF, VLAN30-SERVERS | HQ-STAFF-net, HQ-SERVERS-net | HTTPS, ICMP | No |
| 2 | AppServer-to-DB | VLAN30-SERVERS, VLAN40-DB | HQ-SERVERS-net, HQ-DB-net | MYSQL, ICMP | No |
| 3 | ATM-to-AppServer | VLAN50-ATM, VLAN30-SERVERS | HQ-ATM-net, HQ-SERVERS-net | HTTPS, ICMP | No |
| 4 | Mgmt-to-Infra | VLAN10-MGMT, VLAN30, 40, 50, 60 | HQ-MGMT-net, all | SSH, HTTPS, ICMP | No |
| 5 | Guest-to-Internet | VLAN70-GUEST, port1 | HQ-GUEST-net, all | HTTP, HTTPS, DNS, ICMP | Yes |
| 6 | Staff-to-Internet | VLAN20-STAFF, port1 | HQ-STAFF-net, all | HTTP, HTTPS, DNS, ICMP | Yes |
| 7 | Internet-to-DMZ | port1, VLAN60-DMZ | all, HQ-DMZ-net | HTTPS, ICMP | No |

## VPN policies (FGT-HQ)

| # | Name | Interfaces (in, out) | Source, Destination | Service | NAT |
| --- | --- | --- | --- | --- | --- |
| 8 | Staff-to-BR-Staff | VLAN20-STAFF, HQ-to-BR | HQ-STAFF-net, BR-STAFF-net | HTTPS, ICMP | No |
| 9 | BR-to-HQ-AppServer | HQ-to-BR, VLAN30-SERVERS | BR-STAFF-net and BR-ATM-net, HQ-SERVERS-net | HTTPS, ICMP | No |
| 10 | Mgmt-to-BR-Mgmt | VLAN10-MGMT, HQ-to-BR | HQ-MGMT-net, BR-MGMT-net | SSH, HTTPS, ICMP | No |

## Branch policies (FGT-BR)

| # | Name | Interfaces (in, out) | Source, Destination | Service | NAT |
| --- | --- | --- | --- | --- | --- |
| 4 | BR-Staff-to-HQ-Staff | BR-to-HQ, VLAN110-STAFF | HQ-STAFF-net, BR-STAFF-net | HTTPS, ICMP | No |
| 5 | Staff-and-ATM-to-HQ-AppServer | VLAN110-STAFF and VLAN120-ATM, BR-to-HQ | all, HQ-SERVERS-net | HTTPS, ICMP | No |
| 6 | HQ-Mgmt-to-BR-Mgmt | BR-to-HQ, VLAN100-MGMT | HQ-MGMT-net, BR-MGMT-net | SSH, HTTPS, ICMP | No |

On the Branch side policy 5 uses "all" as the source address. That is safe because the incoming interface is already limited to the staff and ATM VLANs, so nothing from another VLAN can match it. The Branch has no policy to the Internet, so all Branch traffic leaves through the tunnel only. The policy numbers on FGT-BR start at 4 because that is how I numbered them when I built them.

## Why broad rules are dangerous

When I first tried to bring the tunnel up I created two allow-everything policies (any interface, any address, any service) on the tunnel interface, only to get IKE negotiating. Once the tunnel worked I deleted them and wrote the specific rules above. In a bank a rule like that turns the VPN into a flat network. If one Branch PC is infected, the attacker can reach every HQ server, including the database, across a tunnel that the firewall trusts completely. Least privilege keeps a problem in one place from becoming a problem everywhere.

# 7. Site-to-site IPsec VPN

## Concepts

A site-to-site VPN joins two networks. The two firewalls are the endpoints, the tunnel is always available, and users need no client software. A bank uses one because Head Office and Branch have to share transaction and customer data over a link that neither of them controls. Encryption means anyone tapping the link sees only ciphertext, and authentication means each side knows who it is talking to. A remote-access VPN is different: it connects one user's laptop to the network after that person logs in. It suits a traveling employee, not two offices.

IPsec builds the tunnel in two phases. In IKE Phase 1 the two firewalls prove who they are to each other (here with a pre-shared key), agree on algorithms and create a secure channel. In Phase 2 they use that channel to agree keys for the actual data and to state which subnets the tunnel protects. Those subnets are called selectors.

I used a route-based VPN. The tunnel is a virtual interface, so routing decides what goes through it and ordinary firewall policies control it, the same way they control any other interface. That matches how the rest of the lab works.

## Configuration

| Setting | FGT-HQ | FGT-BR |
| --- | --- | --- |
| Phase 1 name | HQ-to-BR | BR-to-HQ |
| Interface | port1 | port1 |
| IKE version | 2 | 2 |
| Peer address | 198.51.100.2 | 203.0.113.2 |
| Proposal | des-sha256 | des-sha256 |
| Diffie-Hellman group | 14 | 14 |
| Authentication | Pre-shared key (identical on both sides, not printed here) | Same key |
| Phase 2 name | HQ-to-BR-p2 | BR-to-HQ-p2 |
| Local selector | 10.10.0.0/16 | 10.20.0.0/16 |
| Remote selector | 10.20.0.0/16 | 10.10.0.0/16 |
| Tunnel interface IP | 172.31.255.1/32 | 172.31.255.2/32 |

## Routing through the tunnel

Each FortiGate has a static route that sends the other site's /16 out of the tunnel interface. Before the VPN existed those routes pointed at ISP-R1. Starting that way let me prove the WAN routing on its own, and then I swapped them once the tunnel was up. Return traffic works because the far side has the mirror route. Static routes are enough for two sites. NAT is not applied to VPN traffic, since the two sites use different private ranges and both firewalls can route to each other's subnets directly.

FGT-HQ # get router info routing-table static

S*   0.0.0.0/0 [10/0] via 203.0.113.1, port1

S    10.20.0.0/16 [10/0] via HQ-to-BR tunnel 198.51.100.2

Two behaviours surprised me and are worth recording. First, FortiOS would not even try to negotiate the tunnel until at least one firewall policy referenced the tunnel interface. The IKE debug output said "no policy configured". Second, a ping to the tunnel interface address itself does not use the tunnel, because that address is outside the Phase 2 selectors.

# 8. How the design protects a bank

| Risk | What in the lab deals with it |
| --- | --- |
| A staff PC is infected and goes after the database | Staff has no rule to the database VLAN. Only the application servers may connect, and only on the MySQL port. |
| Guest devices probe the internal network | The guest VLAN has one rule, out to the Internet. It has no path to any internal or Branch subnet. |
| The public web server is compromised | The DMZ accepts inbound HTTPS only and has no rule to inner VLANs, so it cannot be used as a stepping stone. |
| An ATM is tampered with | The ATM VLAN reaches the application servers on HTTPS and nothing else. |
| Someone listens on the link between sites | All HQ to Branch traffic is encrypted and authenticated by IPsec. |
| Unauthorised administration | Only the management VLAN has SSH and HTTPS to infrastructure. The WAN interfaces answer ping and nothing else. |
| Probing the firewall from outside | Traffic aimed at the firewall's own interface addresses is dropped by the default local-in policy. I saw this in a debug trace. |
| No record of what happened | Every policy logs every session, with the policy name and, for VPN traffic, the VPN type. |
| A broad rule quietly widens access | The allow-everything VPN rules were replaced with specific ones. |

# 9. Testing

## Basic connectivity

FGT-HQ pinged ISP-R1 (203.0.113.1) and FGT-BR pinged ISP-R1 (198.51.100.1), with all five packets returned each time. After the VLAN interfaces were created, every host reached its own gateway.

## Segmentation and VPN

All of these tests used ping, so they show what the firewall permits at the network level. The real-protocol tests follow in the next part.

| # | Test | Expected | Result |
| --- | --- | --- | --- |
| 1 | STAFF-PC1 to APP-SRV | Allow | Replies received |
| 2 | STAFF-PC1 to DB-SRV | Deny | Timeout |
| 3 | GUEST-PC1 to 8.8.8.8 | Allow, NAT | "Destination host unreachable" from ISP-R1, so the packet left the firewall. ISP-R1 has no real Internet route. |
| 4 | GUEST-PC1 to STAFF-PC1 | Deny | Timeout |
| 5 | ATM-PC1 to APP-SRV | Allow | Not recorded in my notes. To be re-run and screenshotted. |
| 6 | ATM-PC1 to DB-SRV | Deny | Timeout |
| 7 | MGMT-PC to APP-SRV and DMZ-WEB | Allow | Replies received from both |
| 8 | DMZ-WEB to STAFF-PC1 | Deny | Timeout |
| 9 | STAFF-PC1 to BR-STAFF-PC1 (VPN) | Allow | Replies, about 63 to 110 ms, TTL 62 |
| 10 | BR-STAFF-PC1 to APP-SRV (VPN) | Allow | Replies received |
| 11 | BR-ATM-PC1 to APP-SRV (VPN) | Allow | Replies received |
| 12 | GUEST-PC1 to BR-STAFF-PC1 | Deny | Timeout |
| 13 | BR-STAFF-PC1 to DB-SRV (VPN) | Deny | Timeout |

## Real HTTPS tests

APP-SRV runs a small Python HTTPS server on port 443 with a self-signed certificate. From MGMT-PC, curl -k https://10.10.30.10 returned the directory listing page, which confirms policy 4 lets real HTTPS through. From BR-STAFF-PC1 at the Branch, wget to the same address returned the same page, and the server's own console logged a GET request from 10.20.110.10 with status 200. That request crossed the IPsec tunnel.

From BR-STAFF-PC1, an HTTPS request to the database VLAN (10.10.40.10) ended in "download timed out". A timeout means the firewall dropped the packets. If they had got through to a host with nothing listening, I would have seen "connection refused" instead. So segmentation holds for real HTTPS across the VPN, not only for ping.

After the tunnel had been idle, the first request sometimes failed with "host unreachable" and the next attempt worked. It looked like the tunnel renegotiating on demand.

I did not manage to test MySQL between APP-SRV and DB-SRV. The package installs inside the containers kept failing (see section 10), so policy 2 has only been checked with ping.

## VPN and log evidence

On FGT-HQ, diagnose vpn tunnel list showed one established security association (sa=1) for HQ-to-BR-p2, with DES and SHA-256 keys in both directions. After the HTTPS test the counters were non-zero (about 48 packets encrypted and 50 decrypted). The security association values on the two firewalls were mirror images of each other.

execute log display on FGT-HQ listed the Branch traffic with policyid=9, policyname="BR-to-HQ-AppServer", action="accept" and vpntype="ipsecvpn". The MGMT-PC HTTPS traffic showed up under policy 4 (Mgmt-to-Infra). For denied traffic, get system session list showed no session at all, which is what I would expect for dropped packets.

I also confirmed that the WAN side cannot reach a firewall's inner interface addresses. A ping from ISP-R1 to 10.20.110.1 failed, and diagnose debug flow on FGT-BR showed a successful route lookup followed by a local-in drop.

# 10. Problems I ran into

These are the issues that cost me time. I list them because the fixes are useful to anyone repeating the lab.

| Problem | Cause | How I found it and what fixed it |
| --- | --- | --- |
| Creating a VLAN interface failed with "Attribute 'vdom' MUST be set" | New interfaces need a VDOM even with VDOMs off | Read the error text. Added set vdom "root" to each interface. |
| Switch VLAN settings did not apply in GNS3 | The fields on the left of the dialog only stage values | The table did not change after Apply. Each port has to be committed with the Add button. |
| FGT-BR port1 would not ping ISP-R1 | I typed 192.51.100.2 instead of 198.51.100.2 | The output said "sendto failed", meaning no matching subnet. Corrected the address. |
| Firewall to firewall ping across sites failed at 100% loss | ISP-R1 only knew its two directly connected links | Added static routes for 10.10.0.0/16 and 10.20.0.0/16 on ISP-R1. |
| Pinging the other site's gateway address failed even with routes in place | That address belongs to the firewall itself, so the packet is handled by local-in policy, not forwarding | diagnose debug flow showed "iprope_in_check() check failed". I tested against real hosts instead. |
| VPCS hosts lost their addresses after a reload | VPCS does not keep settings unless told to | Ran save on each host after setting its IP. |
| Phase 1 proposal commands were rejected | The console kept giving parse errors. The image only ever showed DES proposals, and AES ones were rejected | After many attempts, typing each command slowly, one line at a time, worked for des-sha256. My supervisor approved continuing with DES. |
| Tunnel IP with a /30 mask was refused | Route-based tunnel interfaces need a /32 | Read the error and changed both ends to /32. |
| diagnose vpn tunnel up HQ-to-BR said "Unknown phase2" | The command wants the Phase 2 name | Used HQ-to-BR-p2. |
| No IKE negotiation at all | No firewall policy referenced the tunnel interface | diagnose debug application ike -1 printed "no policy configured". Added tunnel policies first, then tightened them. |
| Adding more than 10 policies failed | The evaluation licence allows 10 policies per VDOM | Read the error. Merged similar rules, for example both Branch source networks in one HQ policy. |
| Containers had two default routes and no Internet | The lab gateway and the temporary NAT gateway both existed | ip route show showed both. Deleted the lab route while installing packages, then added it back. |
| The Python HTTPS server crashed with an ssl error | ssl.wrap_socket was removed in newer Python (3.14 here) | Rewrote the script with ssl.SSLContext and load_cert_chain. |
| Docker containers lost their IP and packages | Runtime network settings and installs did not survive a container reset | Re-applied the IP and route. I have not found a fix that makes it permanent. |
| apk add failed with TLS errors on DB-SRV | Never fully diagnosed. Possibly the container clock or the NAT path | I stopped here and dropped the MySQL test. |

# 11. Limitations of the lab

- DES encryption. The FortiOS image only accepted DES based proposals. DES is obsolete and I would not use it for a real bank. A production tunnel should use AES-256 with SHA-256 or stronger. Changing to AES on a standard image is a change to the proposal line on each side. My supervisor knows and approved DES for this lab.

- ICMP in every policy. ICMP was added so VPCS could test the rules. It should be removed in a real ruleset.

- Ten policy limit. The evaluation licence capped the rule count, so I merged some flows into shared rules instead of writing one rule per flow.

- Some rules were not exercised. Policy 2 (MySQL) was tested with ping only. Policy 10 and Branch policy 6 (management to Branch management) were not tested because I placed no host on the Branch management VLAN. Policy 7 (Internet to DMZ) was not tested from an outside client.

- Switches. The GNS3 built-in switch has no CLI, so I could not show Cisco VLAN commands or restrict which VLANs a trunk carries. A production switch would limit the trunk to the VLANs it needs.

- Test web server. I started the Python server from the container's root directory, so the directory listing exposes the whole filesystem, including the private key. That is fine for a throwaway lab and would never be acceptable for a real application.

- Pre-shared key. A bank would normally use certificates instead of a shared password between the two firewalls.

- Logs. There is no log disk, so logs live in memory and disappear on reboot.

# 12. Conclusion

I built a two site bank network in GNS3 and got the three main parts working together. VLAN segmentation splits the network into zones. Least-privilege firewall policies decide which zone may reach which, and everything else is dropped. An IPsec tunnel between two FortiGates carries Head Office to Branch traffic encrypted. I tested this with ping across all the zones and with real HTTPS across the tunnel, and I collected tunnel counters and log entries as evidence.

The lab has real limits, listed above, and I would not present it as production ready. The main things to fix before anything like it went live are the DES cipher, the ICMP allowances and the untested rules. What I got out of the project is a better understanding of why a firewall rule has to be written for a reason, and how much of the work in a VPN goes into routing and policy, not only the tunnel settings.

# Appendix A. Presentation script

This is what I plan to say when I present to my supervisor. It takes about three minutes.

My project was to design and test the network security for a small bank with a Head Office and a Branch, using firewall rules, network segmentation and a site-to-site VPN. I built it in GNS3 using two FortiGate firewalls, a Cisco router acting as the Internet, and switches.

I started by choosing what each image was for. The FortiGate image ran twice, once for each site. The Cisco 2691 is the Internet in the middle. I did not use the 7200 router or the IOU switch, because I did not need the router and I had no licence for the switch.

The main design decision is that the firewall is the gateway for every VLAN. That means anything moving between VLANs has to pass a firewall rule. At Head Office I have seven VLANs: management, staff, servers, database, ATM, DMZ and guest. The Branch has management, staff and ATM.

The rules follow least privilege. Staff can reach the application servers over HTTPS, but never the database. Only the application servers can talk to the database. The ATM network only reaches the application servers. Guests can only go to the Internet. The DMZ cannot start connections into the inner network. Everything that is not allowed is dropped and I confirmed that with tests.

The two sites are joined by an IPsec tunnel between the two firewalls. It is route-based, so the firewall routes the other site's addresses into the tunnel interface, and normal policies control what goes through. I first tested the routing before the tunnel existed, so I could tell routing problems from VPN problems.

To test, I ran thirteen connectivity tests for allowed and denied traffic, and everything behaved as designed except one test I still need to repeat. Then I used real HTTPS. A Branch computer fetched a page from the HQ server through the tunnel, and the HQ firewall logs show that traffic as IPsec VPN traffic under the correct policy. When the same Branch computer tried the database VLAN, it timed out, which shows the firewall dropped it.

There are limits. The image only allowed DES for encryption, so a real bank would need AES. I also had to allow ping in the rules to be able to test them, and the evaluation licence limited me to ten policies. I could not finish the MySQL test because of package problems in the container.

The biggest lessons for me were how to read the firewall's own debug output to find where a packet stops, and that a VPN is only as good as its routing and policies. Thank you. I am happy to answer questions or show any part of the lab.

# Appendix B. Commands used to verify the lab

| Device | Command | What it shows |
| --- | --- | --- |
| FortiGate | get router info routing-table all | Connected, static and tunnel routes |
| FortiGate | show firewall policy | All rules with interfaces, addresses, services |
| FortiGate | show vpn ipsec phase1-interface | Phase 1 settings for comparing both sides |
| FortiGate | diagnose vpn tunnel list | Security associations, selectors, packet counters |
| FortiGate | get vpn ipsec tunnel summary | Short tunnel status |
| FortiGate | diagnose debug application ike -1 | IKE negotiation step by step |
| FortiGate | diagnose debug flow filter, then trace start | What the firewall does with a packet |
| FortiGate | diagnose sniffer packet <interface> "filter" 4 | Packets on the wire |
| FortiGate | get system session list | Active sessions |
| FortiGate | execute log display | Traffic logs with policy name and VPN type |
| Cisco | show ip interface brief | Interface status and addresses |
| Cisco | show ip route static | Static routes on ISP-R1 |
| Alpine host | ip addr show, ip route show | Address and routes inside a container |